CHAPTER 3

MODELING in the TIME DOMAIN

3.1 INTRODUCTION

2 Approaches for the ANALYSIS and DESIGN of FEEDBACK Control Systems.

- Freq.-Domain (Classical)

Advantages : - RAPIDLY provide STABILITY &Transient Response Information.

Disadvantages : - Can be APPLIED ONLY to LINEAR, Time-Invariant Sys.

- Time-Domain (Modern, State-Space)

+Method for MODELING, ANALYZING, and DESIGNING a wide range of Sys.

+CAN be used to :

- Represent Nonlinear Sys. That have BACKLASH, Saturation, and Dead Zone.

- Sys. With Nonzero Initial Condition.

- Time-Varying Sys.

- Multiple-Input/Output Systems,

Can be represented with a model similar in form and complexity to that used for single input/output Sys.

- Sys. With a Digital Coumputer in the LOOP or to model sys. For digital simulation.

Sys. Response can be obtained for Changes in sys. Parameters – (An important design tool).

- The SAME class of systems modeled by the Classical approach.

3.2 SOME OBSERVATION

Some Following Approach :

- State Variables,

- State Equations,

- If the System has initial condition at ALL Variables, then for the State Variables t ≥ t0

- Output Equation,

- State-Space Representation

Follow these STEPS for STATE-SPACE…

First-Order System STEPS,

- Select the Current (Direction), i(t),

- Write the LOOP Equation,

- Taking The LAPLACE TRANSFORM, and its INITIAL CONDITION

- SOLVE for all the Other Network Variables ALGEBRAICALLY . The Ouput Equation.

- Solve The STATE-SPACE REPRESENTATION.

Second-Order System STEPS

- First-Order Differential is NEEDED for SOLVE 2 State Variables,

- Writing the LOOP Equation, Converting the State Variable,

- Using the LAPLACE TRANSFORM and the METHOD of CHAPTER 2 (Freq.-Domain)

- Find The Output Equation.

- Solve State-Space Representation

+A Second-Order System REQUIRE a MINIMUM  of TWO State Variables to DESCRIBE it,

+The State and Output Equation can be written in VECTOR-MATRIX form, IF THE SYSTEM is LINEAR.

3.3 THE GENERAL STATE-SPACE REPRESENTATION

+Formalize for SOME Definitions;

- Linear Combination,

- Linear Independence,

(Var. Xi, for i = 1 to n) = LINEARly Independence; their S = 0 ⇔ every Ki = 0 and NO Xi = 0 for ALL t ≥ 0.

- Sys. Variable, any var. that RESPONDS to an INPUR or INITIAL COND.,

- State Variable,

- State Vector,

- State-Space,

- State Equation,

- Output Equation

+EQUATION that Represents a System in STATE SPACE ;

+ As an Example,

with x1 and x2 are the State Variables.

If there are a SINGLE OUTPUT, then

3.4 APPLYING THE STATE-SPACE REPRESENTATION

LINEARLY INDEPENDENT STATE VARIABLES

+The Comp. of the State Vector MUST BE Linearly Independent,

MINIMUM NUMBER OF STATE VARIABLES

+The Minimum Number required equals the Order of the Differential Equation Describing the System,

+ Too FEW the State Variables are SELECTED > it may be IMPOSSIBLE to WRITE PARTICULAR OUTPUT Equations,

+You SELECT the MINIMUM NUMBER of STATE VARIABLES but They ARE NOT LINEARLY INDEPENDENT > you MAY NOT BE ABLE to SOLVE for ALL OTHER SYSTEM VARIABLES

+ we want DISPLACEMENT (other STATE VARIABLES) as an OUTPUT > The DENOMINATOR or Characteristic Equation, has INCREASED ORDER to 2 or The Order Initial T.F. (The PRODUCT of 2 TRANSFER FUNCTION),

+The STEPS for ELECTRICAL NET.:

- LABEL ALL,

- SELECT the State Var.by Writing the Derivative Eq. for ALL (ENERGY-Storage elements),

+ Choose the State Var.as the Quantities that are DIFFERENTED,

- APPLY network THEORY,

- SUBTITUTE the RESULT,

- FIND the OUTPUT Eq., & Representing it to VECTOR-MATRIX form (for the STATE-SPACE Representation)

3.5 CONVERTING A TRANSFER FUNCTION TO STATE SPACE

+Generally, These are the STEPS :

- SELECT a SET of STATE VAR. (Phase Variable)

- CONSIDER the DIFFERENTIAL Equation,

- CHOOSE STATE VAR is to choose the output, y(t) & its (n-1) derivatives as the STATE VAR. (Phase Var. Choice)

- DIFFERENTING both sides, the dot = differentiation respect to time

- SUBTITUTING the Definition, the State Eq. are evaluated as

- In VECTOR MATRIX Form,

- Since the SOLUTION to Differential Eq is y(t), or x1, the OUTPUT EQ. is

+ZERO >>>Output Eq.

+POLE>>>State Eq.

3.6 CONVERTING FROM STATE SPACE TO A TRANSFER FUNCTION (SS2TF)

+Given The State Eq and Output Eq.

+Formula SS2TF,

+The STEPS:

- >>>>>>

- >>>Formula SS2TF,

3.7 LINEARIZATION

+THE STEPS :

- WRITING the Differential Eq.

- LETTING

- LINEARIZE X2,

- SUBTITUTING theYIELD of LINEARIZE and DIFFERENTIAL Eq.

- SUBTITUTING the VALUE that DIKETAHUI

- SELECTing the STATE VAR.,

- WRITIING the State and Output Eq.

- CONVERTing to VECTOR-MATRIX form